If you look in a components catalogue you will discover that the resistor values appear to have been chosen by a musician! The 'notes' on the resistor scale are:- 

1.0, 1.2, 1.5, 1.8, 2.2, 2.7, 3.3, 3.9, 4.7, 5.6, 6.8, 8.2 and 10.0
× a power of ten. 
These values are 'equally spaced' in the same way that notes on a (correctly tuned!) piano are equally tempered. You can calculated each subsequent value, [image: image10.png]


approximately, by multiplying the last value by: 

It is called the 'E12' series because there are 12 values, spaced exponentially across a change in value of a power of ten. 

When an engineer designs a circuit, they work out the component values which make it work as well as possible. This may lead to values like 149 Ohms or 255,334 Ohms. The manufacturer then usually chooses the closest value from the E12 series. Instead of 149 and 255,334 Ohms they would use 150 and 27k. Since each E12 value is about 20% bigger than the last, the nearest E12 is never in error by more than about 10%. In practice this kind of error in resistor values rarely affects the behaviour of a circuit. The large quantity of E12s used means that their price is low, keeping manufacturers happy!

Resistance value is marked on the resistor body. The first three bands provide the value of the resistor in ohms and the fourth band indicates the tolerance. Tolerance values of  5%, 2%, and 1% are most commonly available. 

The following table shows the colors used to identify resistor values:

	COLOR
	DIGIT
	MULTIPLIER
	TOLERANCE
	TC

	 Silver
	 
	 x 0.01 W
	±10%
	 

	 Gold
	 
	 x 0.1 W
	±5%
	 

	 Black
	0
	 x 1 W
	 
	 

	 Brown
	1
	 x 10 W
	±1%
	±100*10-6/K

	 Red
	2
	 x 100 W
	±2%
	±50*10-6/K

	 Orange
	3
	 x 1 kW
	 
	±15*10-6/K

	 Yellow
	4
	 x 10 kW
	 
	±25*10-6/K

	 Green
	5
	 x 100 kW
	±0.5%
	 

	 Blue
	6
	 x 1 MW
	±0.25%
	±10*10-6/K

	 Violet
	7
	 x 10 MW
	±0.1%
	±5*10-6/K

	 Grey
	8
	 x 100 MW
	 
	 

	 White
	9
	 x 1 GW
	 
	±1*10-6/K


  ** TC - Temp. Coefficient, only for SMD devices

[image: image1.jpg]For E12 and E24 series For E48 and E96 series

e e
L e
== ==
Y _gue—

Tolerance.

A-Fustagt
] b iSedan
i

E e PN





Fig. 1.2: b. Four-band resistor, c. Five-band resistor, d. Cylindrical SMD resistor, e. Flat SMD resistor
The following shows all resistors from 1R to 22M: 
1R0 = 1.0Ω, 4R7 = 4.7 Ω, 1k2=1.2 Ω,
	[image: image2.png]R 11K 3
w2}
rs [N 3
®e I}
R2 [0}
2R7 IR}
w1 )
aRg WD
LCARLL
586 1R )
org ([0 }
or 1





	[image: image3.png]108 I
2R THIY
15R [T
18R I
2RI[IE
2R I
3R
3R 1IN
4RI
s6R JII%
s8R II¥
a2 JII¥

[ R




	[image: image4.png]100R 1%
120R [[II¥
150R JEI%
180R 1%
2208 [[TI¥
270R (1K
330R 1K
390R 1K
470R [ 1K
560R [TIX
680R 11V
820R [JII%

[ R




	[image: image5.png]ﬂkZ T






 

	[image: image6.png]10k I
12k |
15k 1
18k |1
2k
27k
3K
39K
47k
s6k 1]
68k I
82k |

[ R




	[image: image7.png]100k J X
1206 I
150k 1
160k 11
220k Ik
270k I
330k (11
LI
470k | 1
g0k 11
sook 11
820k [





	[image: image8.png]o JII }
w2 JII }
s [T}
g [T}
am2 [}
2m7 ][]l )

s 1T )
omg TEIT )
o2 I




	[image: image9.png]i I}

L L
2






